SIMATS ENGINEERING

CO- 2: AT – 2: Simulation Task - Medium

COURSE NAME: 5G / 6G Communication Systems

COURSE CODE: ECA56

COURSE OUTCOME COVERED

CO: 2 - Analyse LTE and 5G wireless network architectures, including carrier  aggregation, MIMO techniques, beamforming strategies, and service-based core  network functional entities. (BL4)

Assessment Tool Weightage: 35%

Short description about assessment: Performance-based evaluation using simulation  tools to apply theoretical concepts practically. This question paper consists of a total  of 40 questions, out of which each student is required to answer any 5 questions. Each  question carries 20 marks, and the paper carries a total of 100 marks

QUESTIONS:5 Total Marks: 100 Duration: 120 Mins

Question 1

Simulation of LTE throughput performance under varying bandwidth, SNR, modulation order, coding rate, and user load. (BL4)

Question 3

Simulation of carrier aggregation performance in LTE-Advanced under single carrier, dual carrier, triple carrier, varying bandwidth, and SNR. (BL4)

Question 6

Simulation of BER performance in 2×2, 4×4, 8×8 MIMO under varying SNR, modulation schemes, fading conditions, and antenna correlation. (BL4)

Question 11

Simulation of beamforming gain under steering angle, antenna elements, frequency, SNR, and interference. (BL4)

Question 31

Simulation of propagation loss under 700 MHz, 3.5 GHz, 28 GHz, 38 GHz, and 60 GHz. (BL4)

These are the five questions for which the MATLAB codes were provided.

# **Q1. Simulation of LTE Throughput Performance under Varying SNR (Question 1)**

```matlab

clc;

clear;

close all;

% LTE Throughput Simulation

SNR = 0:2:30;          % dB

BW = 20e6;             % 20 MHz

Throughput = BW .* log2(1 + 10.^(SNR/10));

figure;

plot(SNR, Throughput/1e6,'-ob','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('LTE Throughput vs SNR');

disp(table(SNR',Throughput'/1e6,...

'VariableNames',{'SNR_dB','Throughput_Mbps'}));

```

---

# **Q2. Simulation of Carrier Aggregation Performance (Question 3)**

```matlab

clc;

clear;

close all;

SNR = 0:2:30;

BW1 = 20;

BW2 = 40;

BW3 = 60;

T1 = BW1*log2(1+10.^(SNR/10));

T2 = BW2*log2(1+10.^(SNR/10));

T3 = BW3*log2(1+10.^(SNR/10));

figure;

plot(SNR,T1,'r','LineWidth',2);

hold on;

plot(SNR,T2,'b','LineWidth',2);

plot(SNR,T3,'k','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Relative Throughput');

title('LTE Carrier Aggregation');

legend('Single Carrier','Dual Carrier','Triple Carrier');

```

---

# **Q3. Simulation of BER Performance in 2×2, 4×4 and 8×8 MIMO (Question 6)**

```matlab

clc;

clear;

close all;

SNR = 0:2:20;

BER_2x2 = 0.5*erfc(sqrt(10.^(SNR/10)));

BER_4x4 = BER_2x2/2;

BER_8x8 = BER_2x2/4;

figure;

semilogy(SNR,BER_2x2,'-or','LineWidth',2);

hold on;

semilogy(SNR,BER_4x4,'-sb','LineWidth',2);

semilogy(SNR,BER_8x8,'-^k','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate');

title('BER Performance of MIMO');

legend('2x2','4x4','8x8');

```

---

# **Q4. Simulation of Beamforming Gain (Question 11)**

```matlab

clc;

clear;

close all;

theta = -90:1:90;

N = 8;

Gain = abs(sin(N*pi*sind(theta)/2)./(N*sin(pi*sind(theta)/2)));

Gain(isnan(Gain)) = 1;

figure;

plot(theta,20*log10(Gain),'LineWidth',2);

grid on;

xlabel('Steering Angle (Degree)');

ylabel('Gain (dB)');

title('Beamforming Pattern');

ylim([-40 5]);

```

---

# **Q5. Simulation of Propagation Loss (Question 31)**

```matlab

clc;

clear;

close all;

distance = 1:0.5:10;      % km

freq = [700 3500 28000 38000 60000];

figure;

hold on;

for i=1:length(freq)

PL = 32.44 + 20*log10(freq(i)) + 20*log10(distance);

plot(distance,PL,'LineWidth',2);

end

grid on;

xlabel('Distance (km)');

ylabel('Path Loss (dB)');

title('Propagation Loss');

legend('700 MHz','3.5 GHz','28 GHz','38 GHz','60 GHz',...

'Location','northwest');

```

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Simulation Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Conceptual Understanding | 20% (4 marks ) | Demonstrates strong understanding of underlying concepts in the simulation | Shows good understanding with minor gaps | Partial understanding; some misconceptions | Lacks understandin g of key concepts |
| Model Design | 25% (5 marks ) | Develops accurate, well structured simulation/mod el independently | Develops correct model with minor errors | Model is incomplete or requires guidance | Unable to develop a proper model |
| Tool Usage | 20% (4 marks ) | Uses simulation tools effectively with correct setup and execution | Uses tools with minor errors or guidance | Limited ability to use tools properly | Unable to use simulation tools |
| Analysis of Results | 20% (4 marks ) | Provides clear, logical analysis and meaningful interpretation of results | Analysis is reasonable with minor gaps | Superficial analysis; limited interpretation | Unable to analyze or interpret results |
| Documentatio n | 15% (3 marks ) | Presents results clearly with proper documentation and structured reporting | Documentation is adequate with minor issues | Poorly organized or incomplete documentation | No proper documentatio n or presentation |